Here’s a clean layout plan for pairing 16 mm marbles with a 60 LEDs/m WS2812B strip (≈ 16.67 mm LED pitch) so that each marble gets its own “pixel” without crowding or gaps.

📐 Core Measurements

| Element | Size / Spacing |
| --- | --- |
| Marble diameter | 16 mm |
| LED pitch (center-to-center) | 16.67 mm |
| Recommended hole center spacing | 16.7 mm |
| Recommended hole diameter | 15.8–16.0 mm |

🛠 Grid Layout Tips

- 1:1 mapping: Align each marble center directly over its LED center.

- Frame thickness: Leave ~0.5–1 mm wall between each hole for strength (material permitting).

- Alignment tolerance: ±0.2 mm is fine — LEDs emit wide enough light to fill marble face.

- Edge spacing: Keep 8–10 mm margin between first marble center and frame edge for wiring.

🔲 Example Panel Dimensions

For a 10 × 10 marble panel:

- Active area = 9 × 16.67 mm horizontal spacing by 9 × 16.67 mm vertical spacing
≈ 150 × 150 mm of marble grid.

- Outer frame (with edge margins) ≈ 166 × 166 mm.

💡 Build Notes

- Light cups: Small reflective wells behind each LED help direct light into marble.

- Diffusion control: For sharper pixel edges, mask or paint backs so only LED-lit face glows.

- Modularity: Keep panel in even increments so you can tile multiple modules seamlessly.

# Hardware Notes

- For reference, the kaleidoscope has 20 x 39 = 780 pixels

- A ‘standard’ marble is 14-16mm in diameter

- A 60 LEDs/m WS2812B strip has a 16.67mm LED pitch

  - Dave’s 3D printer has a print volume of 350 x 350 x 325 mm or 20 x 20 = 400 marbles.

## Power

- usb c pd adjustable power supply

# Software Notes

- marble clock

- marble roller

- Display static patterns/images

- Display dynamic patterns/images

- Animated transitions

  - Try alpha blending between marbles in motion (ala Matrix/Kaleidoscope) to give a better sense of motion. Look at “lerp” (ask AI).

  - Marbles ‘fall’ from the top down in a random pattern ala connect 4 (need a physics engine?)

  - Marbles ‘fall’ out the bottom ala Connect 4

  - Marbles ‘roll’ down from row to row filling in the picture

  - Standard transitions:

    - Simple Cut

    - Fade to/from black

    - Alpha blend to next picture

    - Wipe

    - Push/slide from left/right/top/bottom - old slide out, new slide in